# Task 1.2: Exploratory Data Analysis

National Electricity Grid Network Analysis — Team Member 2 (Data Analyst)

# Objective

Understand the characteristics of the utilities, substations, and lines datasets, and identify initial patterns in the Ghanaian grid network to guide the deeper network and business-intelligence analysis in Week 2.

# 1. Dataset Overview

The dataset was generated using the seeded script provided in the project brief (random.seed(42)), producing three related tables:

- utilities.csv — 10 rows (electricity utilities operating in Ghana and neighbouring countries)

- substations.csv — 44 rows (grid substations across Ghana's regions plus cross-border interconnection points)

- lines.csv — 55 rows (transmission/distribution lines connecting substations)

# 2. Descriptive Statistics — Numerical Variables

Substations:

| Metric | Voltage (kV) | Capacity (MVA) | Commissioning Year |
| --- | --- | --- | --- |
| Mean | 134.5 | 157.9 | 1996.3 |
| Std Dev | 120.4 | 139.9 | 16.1 |
| Min | 11 | 6.4 | 1967 |
| Median | 69 | 108.6 | 1999.5 |
| Max | 330 | 487.6 | 2022 |

Lines:

| Metric | Voltage (kV) | Length (km) | Capacity (MVA) |
| --- | --- | --- | --- |
| Mean | 141.4 | 99.3 | 222.4 |
| Std Dev | 135.2 | 90.3 | 109.2 |
| Min | 11 | 3.8 | 32.9 |
| Median | 69 | 75.9 | 229.9 |
| Max | 330 | 426.0 | 506.3 |

# 3. Frequency Distributions — Categorical Variables

- Substation Type: Transmission (21), Distribution (15), Bulk Supply Point (8)

- Substation Status: Active (43), Inactive (1)

- Line Status: Active (53), Under Maintenance (2)

- Line Type: Overhead (40), Underground (15)

- Greater Accra has the most substations (6), followed by Ashanti (5); several cross-border regions have exactly 1 each

# 4. Top Utilities by Number of Lines Operated

| Utility | Type | Line Count |
| --- | --- | --- |
| GRIDCo | Transmission | 24 |
| NEDCo | Distribution | 14 |
| ECG | Distribution | 10 |
| CEB | Transmission | 3 |
| CIE | Distribution | 2 |
| SONABEL | Distribution | 2 |

GRIDCo, Ghana's national transmission utility, operates nearly twice as many lines as the next-largest operator (NEDCo), which is expected given its role as the backbone transmission company connecting regions and borders.

# 5. Most-Connected Substations

| Substation | Region | Voltage (kV) | Connections |
| --- | --- | --- | --- |
| Kumasi Central | Ashanti | 161 | 5 |
| Mallam | Greater Accra | 330 | 5 |
| Cape Coast | Central | 69 | 5 |
| Tema | Greater Accra | 330 | 4 |
| Legon | Greater Accra | 161 | 4 |

Kumasi Central, Mallam, and Cape Coast are the network's most-connected 'hub' substations, each with 5 direct lines — these are strong early candidates for the critical-substation analysis in Task 2.1.

# 6. Geographic Distribution

Greater Accra leads in both substation count (6) and line count (11 lines by source substation), consistent with it being Ghana's most urbanized, highest-demand region. Ashanti (Kumasi area) is the second-largest hub. Upper West and most cross-border points have only a single substation each, suggesting thinner, more fragile connectivity at the network's edges.

# 7. Voltage-Level Distribution

Substations are fairly spread across voltage tiers: 11kV (9), 33kV (6), 69kV (8), 161kV (11), 330kV (10) — the 161kV and 330kV transmission tiers together make up nearly half of all substations, reflecting the transmission-heavy nature of the synthetic network.

# 8. Visualizations

# 9. Initial Hypotheses About Network Structure

- Greater Accra and Ashanti will likely dominate centrality metrics in Task 2.1, since they have the most substations and connections.

- Border-interconnection substations, despite high voltage/capacity, may act as structural bottlenecks (single points of connection) — worth checking in the N-1 contingency analysis.

- GRIDCo's heavy involvement in inter-regional backbone lines suggests it will score highest on betweenness centrality, not just degree centrality.

# 10. Patterns for Further Investigation

- Only 1 substation is Inactive and only 2 lines are Under Maintenance — worth flagging as a reliability question in Task 2.3 (is this realistic, or does it understate maintenance backlog?)

- Wide spread in line length (3.8km to 426km) — categorizing into short/medium/long transmission runs (Task 2.2) may reveal regional vs. cross-border patterns.

- Capacity variation is large (6.4–487.6 MVA) — worth checking which substations are operating close to rated capacity as upgrade candidates (Task 2.3).